FAULT DIAGNOSIS

3.0 	GENERAL

- The Radio set being modular in construction, fault can be easily traced and pin pointed by isolating the faulty module whenever the set fails to operate. The repair of a faulty set can be performed only with good background knowledge of the working of the set.

- Whenever the Receiver Transmitter develops a fault, the troubleshooting procedure given below may be followed. Many symptoms are discussed along with probable faulty module/component. If a module is suspected it can be tested, before replacing it with a Spare module. The objective is to make sure that the module in question is indeed faulty before replacing and to avoid arbitrary replacement of modules. The faulty module can be then thoroughly checked by referring to the Technical Mechanical Part III Overhaul and Reconditioning Instructions.

### CAUTION

ENSURE SET IS SWITCHED OFF WHILE REPLACING MODULES/CONNECTOR DURING TROUBLESHOOTING OF THE RADIO UNDER TEST.

3.1     TROUBLESHOOTING

The following procedure will enable to quick the effective repair of  Secure HHPC , to identify the faulty module. These methods provide an elementary fault finding procedure. It is not exhaustive and may be used as broad guidelines only.

3.1.1   TROUBLESHOOTING WITH THE HELP OF FLOWCHARTS

1. The troubleshooting has been covered systematically system wise and then card/Module wise. The following layout for the fault diagnosis is to be followed.

Procedure-1:  Dead unit troubleshooting flowchart : This procedure is to followed when Radio Set is dead.

Procedure-2: BITE check : This procedure is to check the failure of any module in BITE.

Procedure-3:Display/Keypad Troubleshooting : This procedure is to locate the fault whenever             any problem with display or keypad.

Procedure-4:Troubleshooting in Receiver path : Whenever there is no reception in the Radio Set, this procedure is to followed.

Procedure-5 Troubleshooting in Transmitter : Whenever there is transmission problem with the radio set, this procedure is followed.

Procedure 1:  Dead unit troubleshooting flowchart

Set not getting ON

Check Bat voltage. It should be charged.

Charge the Battery

No

Yes

Battery connection issue.

Check Voltage at TP 34. It should be ~7.4V/bat voltage

No

Yes

Check Voltage at TP 35 – 3.6 V                        TP 36 – 5 V

U24 Faulty

No

Yes

Check voltage at R55. It should be 3.3V

U29 Faulty

No

Yes

Display Faulty. Check display using test jig.

Procedure-2: BITE check: This procedure is to check the failure of any module in BITE.

START

Connect battery to the Radio Set. Connect Antenna to the Radio. Switch on the Radio Set and then select list and select iBITE

Does Display

Shows BB:OK’

In Baseband card reprogrammed SCT

RTC may be Faulty in SM Card

SPI flash may be faulty in SM  Card

Does Display

shows ‘R:1’

Does Display

shows ‘S:1’

FPGA may be faulty in

SM card

Does Display

shows ‘F:1’

Check RF path and Reprogrammed SCT

Does Display

Shows ‘RF:OK’

Procedure-3: Display/Keypad Troubleshooting: This procedure is to locate the fault whenever any problem with display or keypad.

Start

Yes

Yes

No

Keypad or Keypad

Cable is faulty.

Select compose (Menu        Message        Compose) and check all keys.

Connect Battery to Radio Set. Connect Antenna to the Radio & Switch ON the Radio.

Does Display shows proper characters

Keypad &

Display OK

Is Digit Display Proper?

Is Display Missing or Unwanted Character Displayed

No

Yes

Check proper connection of display cable

Or Display Faulty

Display Ok

Procedure-4: Troubleshooting in Receiver path: Whenever there is no reception in the Radio Set, this procedure is too followed.

Start

No

No

No

TC XO

Faulty

Check freq

38.4 MHz at

U38

Check Speaker

Check audio at C21

Display RSSI sign on RF on

Bad SINAD

No recovered audio

Yes

Yes

Check L.O Freq

At C114 = Double the channel freq

No

Yes

U21

Faulty

Check voltage at

L37 = 1.2V

Yes

U4

Faulty

TX not working

Procedure-5 Troubleshooting in Transmitter: Whenever there is transmission problem with the radio set, this procedure is followed.

SW 5

Faulty

Check 38.4 MHz

At L42

Controller Faulty

PTT ON

U4

Faulty

U38

Faulty

No

No

Check toggling at R179 on PTT pressing

No

Check toggling at R248 on PTT pressing

No

No

Check shorting of SW 5 when pressed

Press PTT RED LED doesn’t glow

U4

Faulty

U21

Faulty

Check voltage at

L37 = 1.2V